DRAFT - not approved. Do not act on this document until a person has reviewed and approved it.

# V-101 shell wall loss at point C

Classification: Internal

To: Plant Manager, Unit 4

Reference: INSP-2026-081

Audience: Plant Manager, Unit 4

Date: 2026-09-30

# Subject

Shell thickness of V-101 at point C after the August UT survey.

# Findings

- The governing reading at point C is 8.2 mm, with pitting noted [M:mi-b80f1c19-cddb-464d-a75f-1e58939c23d3@1].

- Where pitting is recorded the minimum allowable thickness is 9.0 mm [M:mi-63fd1981-1805-46b9-8318-f0139ce3ed02@1].

# Calculation

Wall loss against the minimum is 8.889 % [C:calc-7cca578871cf8b51].

# Recommendation

Re-survey point C within 14 days and hold V-101 below its reduced operating limit until then [M:mi-69b16ad5-81ea-4321-be7c-a8a37a3ac8e3@1].

# Assumptions

The survey instrument was within its calibration interval; no other assumption is made.

# Supporting references

- [C:calc-7cca578871cf8b51] calc-7cca578871cf8b51: wall_loss = (t_min - t_meas) / t_min = 8.889 % [computed] from t_min = 9.0 mm [M:mi-63fd1981-1805-46b9-8318-f0139ce3e

- [M:mi-63fd1981-1805-46b9-8318-f0139ce3ed02@1] memory item mi-63fd1981-1805-46b9-8318-f0139ce3ed02@1: SOP-114 rev D section 3.1: minimum allowable shell thickness for V-101 where pitting is re

- [M:mi-69b16ad5-81ea-4321-be7c-a8a37a3ac8e3@1] memory item mi-69b16ad5-81ea-4321-be7c-a8a37a3ac8e3@1: Re-survey point C within 14 days and hold V-101 below its reduced operating limit until th

- [M:mi-b80f1c19-cddb-464d-a75f-1e58939c23d3@1] memory item mi-b80f1c19-cddb-464d-a75f-1e58939c23d3@1: UT survey of V-101, August: governing shell reading at point C is 8.2 mm, pitting noted

# How this was produced

Task r · template inspection_approval_note@1 (definition sha-256 c3cad672d378) · content by fixture-model · figures from ARJUN's calculation engine, cited by record · DRAFT — task r · 2026-09-30T13:03:12.041193791+00:00 · model fixture-model · Internal

Produced by ARJUN for task r at 2026-09-30T13:03:12.041193791+00:00 using fixture-model. Content supplied by the named model; structure and required fields enforced by the template engine.. DRAFT — not verified; do not act on this document until reviewed.